Merten 506004 – Configuration Bitmask Technical Note

Reverse-Engineered Behavior of Parameter 1 (Mode Selection)

Introduction

Author: Søren Device: Merten CONNECT / Z-Wave 506004 Wall Transmitter Purpose: Document the undocumented bitmask logic used by CONNECT and Z-Wave modes.

1. Overview

The Merten 506004 wall transmitter operates with a single configuration parameter, Parameter 1, which dictates its operating mode. While the official documentation specifies only a handful of fixed values—0, 4, 44, 52, 54, and 55—the device’s configuration actually leverages a bitmask. Each bit in this mask enables or alters a particular feature or behavior of the transmitter.

Devices configured via CONNECT often use values that do not appear in the public Z-Wave database. One example is value 22 (0x16), which is set when using the legacy Merten CONNECT software.

This technical note provides an explanation for the meaning of each bit and presents a consolidated table of known parameter values.

2. Bitmask Structure

Parameter 1 is an eight-bit value:

Code

Bit:   7 6 5 4 3 2 1 0

Value: 128 64 32 16 8 4 2 1

Each bit activates a specific function. The interpretations below are based on information from public Merten documentation, Z-Wave behavior, CONNECT behavior, and the reverse-engineering of actual devices.

- Public Merten documentation

- Z-Wave behavior

- CONNECT behavior

- Reverse-engineering of real devices

3. Bit Meaning Table (Reverse-Engineered)

| Bit | Value | Meaning (observed) |
| --- | --- | --- |
| 0 | 1 | “Raise shutter” / special action in shutter mode |
| 1 | 2 | Dual-event mode (press + release) / enable key surface |
| 2 | 4 | Switching function enabled (present in all switching modes) |
| 3 | 8 | Doorbell / momentary mode |
| 4 | 16 | Toggle behavior / single-surface mode |
| 5 | 32 | Multi-channel encapsulation (endpoint-based commands) |
| 6 | 64 | Undocumented (unused in known modes) |
| 7 | 128 | Undocumented (unused in known modes) |

Bits 6 and 7 have not been observed in any CONNECT or Z-Wave configuration.

4. Known Parameter Values and Their Meaning

Combined Table

| Decimal | Hex | Binary | Bits Set | Meaning |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | – | Switching/dimming/shutters dual-surface (default Z-Wave) |
| 4 | 4 | 100 | 2 | Switching single-surface |
| 22 | 22 | 10110 | 1,2,4 | CONNECT-only mode: toggle on all keys |
| 44 | 44 | 101100 | 2,3,5 | Doorbell function |
| 52 | 52 | 110100 | 2,4,5 | Lower roller shutter (hold) |
| 54 | 54 | 110110 | 1,2,4,5 | Move roller shutter single-surface |
| 55 | 55 | 110111 | 0,1,2,4,5 | Raise roller shutter (hold) |

5. Analysis of Patterns

5.1 Documented Values Follow a Clear Bit Pattern

- Bit 2 (4) → always present in switching/shutter modes

- Bit 5 (32) → always present in shutter modes

- Bit 3 (8) → only present in doorbell mode

- Bit 0 (1) → only present in “raise shutter”

- Bit 4 (16) → toggle / single-surface behavior

5.2 The Undocumented Value 22 (0x16)

22 = 16 + 4 + 2 → bits 4 + 2 + 1

This combination does not appear in the public Z-Wave database, but it fits the observed pattern:

- Bit 4 (16) → toggle behavior

- Bit 2 (4) → switching function

- Bit 1 (2) → dual-event / enable key surface

This matches the CONNECT behavior:

“Toggle on all keys” mode used by Merten CONNECT.

CONNECT software automatically writes this value when configuring the transmitter.

6. Practical Implications

6.1 CONNECT-Specific Behavior

CONNECT uses custom bitmask values (such as 22) to simulate “virtual wiring.” In this configuration:

- Buttons transmit Multi Channel BASIC_SET commands

- Endpoints are assigned to receivers

- No SCENE_ACTIVATION is used

- Behavior deviates from standard Z-Wave controllers

6.2 Recreating CONNECT Behavior in a Modern Z-Wave System

To emulate CONNECT behavior:

- Set Parameter 1 = 22

- Enable Multi Channel associations

- Monitor for frames such as 60 06 01 20 01 FF (encapsulated BASIC_SET)

This approach restores the familiar CONNECT “toggle on press” behavior.

7. Conclusion

The Merten 506004 wall transmitter uses a versatile bitmask configuration system that was never fully documented. CONNECT leverages custom combinations—most notably value 22—to implement its unique virtual wiring model.

By understanding this bitmask logic, users can:

- Recreate CONNECT behavior with modern Z-Wave controllers

- Debug legacy installations

- Migrate from CONNECT to standard Z-Wave

- Prevent loss of functionality during system upgrades

This document preserves the reverse-engineered knowledge to ensure it remains available for future reference.